- Section 099000
Painting and Coating - Commercial Guide Spec – PPG

PART 1  GENERAL

SECTION INCLUDES

Surface preparation and field painting of exposed interior items and surfaces.

Surface preparation and field painting of exposed exterior items and surfaces.

Surface preparation and field application of interior high-performance coating systems to items and surfaces scheduled.

Surface preparation and field application of exterior high-performance coating systems to items and surfaces scheduled.

Painting of exposed bare and covered pipes and ducts, hangers, exposed steel and iron supports, and surfaces of mechanical and electrical equipment that do not have a factory-applied final finish.

RELATED Requirements

Section 033000 - Cast-in-Place Concrete.

Section 051200 - Structural Steel Framing.

Section 055000 - Metal Fabrications.

Section 062000 - Finish Carpentry:  Shop priming architectural woodwork.

Section 081113 - Hollow Metal Doors and Frames.

Section 092116 - Gypsum Board Assemblies.

DEFINITIONS

General:  Standard coating terms defined in ASTM D16.

Flat refers to a lusterless or matte finish with a gloss range below 15 when measured at an 85 degree meter.

Eggshell refers to low-sheen finish with a gloss range between 20 and 35 when measured at a 60 degree meter.

Semi-Gloss refers to medium-sheen finish with a gloss range between 35 and 70 when measured at a 60 degree meter.

Full gloss refers to high-sheen finish with a gloss range more than 70 when measured at a 60 degree meter.

Environments:  The following terms distinguish between different corrosive exposures:

"Severe environments" are highly corrosive industrial atmospheres with sustained exposure to high humidity and condensation and with frequent cleaning using strong chemicals. Environments with heavy concentrations of strong chemical fumes and frequent splashing and spilling of harsh chemical products are severe environments.

"Moderate environments" are corrosive industrial atmospheres with intermittent exposure to high humidity and condensation, occasional mold and mildew development, and regular cleaning with strong chemicals. Environments with exposure to heavy concentrations of chemical fumes and occasional splashing and spilling of chemical products are moderate environments.

"Mild environments" are industrial atmospheres with normal exposure to moderate humidity and condensation, occasional mold and mildew development, and infrequent cleaning with strong chemicals. Environments with low levels of mild chemical fumes and occasional splashing and spilling of chemical products are mild environments. Normal outdoor weathering is also considered a mild environment.

REFERENCE Standards

ASTM D16 - Standard Terminology for Paint, Related Coatings, Materials, and Applications 2016.

SSPC-SP 6 - Commercial Blast Cleaning 2007.

SSPC-SP 10 - Near-White Blast Cleaning 2007.

SUBMITTALS

See Section 013000 - Administrative Requirements for submittal procedures.

Product Data:  For each paint system indicated, including:

Material List:  An inclusive list of required coating materials. Indicate each material and cross reference specific coating, finish system, and application. Identify each material by manufacturer's catalog number and general classification.

Preparation instructions and recommendations.

Manufacturer's Information:  Manufacturer's technical information, including label analysis and instructions for handling, storing, and applying each coating material.

Selection Samples:  For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

Verification Samples:  For each finish product specified, two samples, minimum size 6 inch (150 mm) square, representing actual product, color, and patterns.

QUALITY ASSURANCE

Installer Qualifications:  A firm or individual experienced in applying paints and coatings similar in material, design, and extent to those indicated for this project, whose work has resulted in applications with a record of successful in-service performance.

Obtain block fillers and primers for each coating system from the same manufacturer as the finish coats.

Paint exposed surfaces. If an item or a surface is not specifically mentioned, paint the item or surface the same as similar adjacent materials or surfaces. If a color of finish is not indicated, Architect will select from standard colors and finishes available.

Do not paint prefinished items, concealed surfaces, finished metal surfaces, operating parts, and labels.

Mock-Up:  Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

See Section 014000 - Quality Requirements, for general requirements for mock-up.

Finish areas designated by Architect.

Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

Refinish mock-up area as required to produce acceptable work.

DELIVERY, STORAGE, AND HANDLING

Deliver materials to project site in manufacturer's original, unopened packages and containers bearing manufacturer's name and label.

Store materials not in use in tightly covered containers in a well-ventilated area at a minimum ambient temperature of 45°F (7°C). Maintain storage containers in a clean condition, free of foreign materials and residue.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

Apply waterborne paints only when temperatures of surfaces to be painted and surrounding air are between 50°F (10°C) and 90°F (32°C).

Apply solvent-thinned paints only when temperatures of surfaces to be painted and surrounding air are between 45°F (7°C) and 95°F (35°C).

Do not apply paint in snow, rain, fog, or mist; or when relative humidity exceeds 85 percent; or at temperatures less than 5°F (3°C) above the dew point; or to damp or wet surfaces.

Painting may continue during inclement weather if surfaces and areas to be painted are enclosed and heated within temperature limits specified by manufacturer during application and drying periods.

EXTRA MATERIALS

Furnish extra paint materials from the same production run as the materials applied and in the quantities described below. Package with protective covering for storage and identify with labels describing contents. Deliver extra materials to Owner.

Quantity:  Furnish Owner with an additional three percent, but not less than 1 gallon (3.8 l) or 1 case, as appropriate, of each material and color applied.

PART 2  PRODUCTS

MANUFACTURERS

Basis of Design Manufacturer:  Pittsburgh Paints, 400 Bertha Lamme Drive Cranberry, PA 16066. Toll-Free Tel:  888-PPG-IDEA. Web:  www.pittsburghpaintsco.com/#sle. Pittsburgh Paints is the exclusive distributor of PPG Protective and Marine Coatings.

PAINT MATERiALS - GENERAL

Material Compatibility:  Provide block fillers, primers, and finish-coat materials that are compatible with one another and with the substrates indicated under conditions of service and application, as demonstrated by manufacturer based on testing and field experience.

VOC Classification:  Provide high-performance coating materials, including primers, undercoats, and finish-coat materials, that meet the applicable local, state, or federal VOC requirements.

Commercial FACILITY INTERIOR PAINT SYSTEMS

Lobby/Common Areas - Gypsum Board Walls:

Acrylic Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 6-411ZV Series Speedhide Zero Interior Latex Eggshell

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-500ZV Series Speedhide Interior Zero VOC Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 9-310XI Series Pure Performance Interior Latex Eggshell.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 9-510XI Series Pure Performance Interior Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

- Alternate High Performance Option

Acrylic Pre-Catalyzed Epoxy Finish: Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Gypsum Board Ceilings:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 9-110XI Series Pure Performance Interior Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Concrete Masonry Units (CMU):

Acrylic Finish:  Two finish coats over a block filler.

Eggshell Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Eggshell Finish:  Pittsburgh Paints; 6-411ZV Series Speedhide Interior Zero VOC Latex Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-500ZV Series Speedhide Interior Zero VOC Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a block filler.

Eggshell Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Eggshell Finish:  Pittsburgh Paints; 9-310XI Series Pure Performance Interior Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 9-510XI Series Pure Performance Interior Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

- Alternate High Performance Option

Acrylic Pre-Catalyzed Epoxy Finish: Two finish coats over a block filler.

Eggshell Finish:

Block Filler:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Block Filler:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Masonry Concrete Ceilings and Soffits:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 9-110XI Series Pure Performance Interior Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Painted Wood; Doors, Frames, Trim, or Chair Rails:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 17-921XI Series Seal Grip 100 Percent Acrylic Universal Primer.

Applied at a dry film thickness of not less than 1.6 mils (0.041 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 17-921XI Series Seal Grip 100 Percent Acrylic Universal Primer.

Applied at a dry film thickness of not less than 1.6 mils (0.041 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Stained Wood; Trim or Chair Rails:

Acrylic Finish:  Two polyurethane coats over stained substrate.

Satin Finish:

Satin Finish:  Pittsburgh Paints; PLX259 Series ProLuxe Interior/Exterior Polyurethane Water Based Acrylic Satin.

Applied at a dry film thickness of not less than 0.8 mils (0.020 mm).

Semi-Gloss Finish:

Semi-Gloss Finish:  Pittsburgh Paints; PLX258 Series ProLuxe Interior/Exterior Polyurethane Water Based Acrylic Semi-Gloss.

Applied at a dry film thickness of not less than 0.8 mils (0.020 mm).

Stained/Gloss Finish:

Gloss Finish:  Pittsburgh Paints; PLX257 Series ProLuxe Interior/Exterior Polyurethane Water Based Acrylic Gloss.

Applied at a dry film thickness of not less than 0.8 mils (0.020 mm).

Lobby/Common Areas - Ferrous Metal Doors, Frames, and Miscellaneous Metals:

Acrylic Finish:  Two finish coats over a primer.

Satin Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Satin Finish:  Pittsburgh Paints; V52-410 Series Break-Through Interior/Exterior Water-Borne Acrylic Satin.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Lobby/Common Areas - Ferrous and Non-Ferrous Metals for High Performance Finish (Including Handrails):

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Lobby/Common Areas - Non-Ferrous Metal; Galvanized Surfaces and Aluminum:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Restaurants/Restrooms/Corridors - Gypsum Board Walls:

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate High Performance Option

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Restaurants/Restrooms/Corridors - Gypsum Board Ceilings:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 9-110XI Series Pure Performance Interior Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Restaurants/Restrooms/Corridors - Concrete Masonry Units (CMU):

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a block filler.

Eggshell Finish:

Block Filler:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils.

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Block Filler:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate High Performance Option

Acrylic Epoxy High Performance Finish:  Two finish coats over a block filler.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Restaurants/Restrooms/Corridors - Masonry Concrete Ceilings and Soffits:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 9-110XI Series Pure Performance Interior Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Restaurants/Restrooms/Corridors - Ferrous Metal - Doors, Frames, and Miscellaneous Metals:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Epoxy High Performance Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Restaurant/Restrooms/Corridors - Ferrous and Non-Ferrous Metals for High Performance Finish (Including Handrails):

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Restaurant/Restrooms/Corridors - Non-Ferrous Metal; Galvanized Surfaces and Aluminum:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior  Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Cubicles/Offices/Conference Room/Break Room - Gypsum Board Walls:

Acrylic Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints:  6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 6-411ZV Series Speedhide Interior Zero VOC Latex Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints:  6-4900XI Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-500ZV Series Speedhide Interior Zero VOC Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Eggshell Finish:  Pittsburgh Paints; 9-310XI Series Pure Performance Interior Latex Eggshell.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 9-900 Series Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 9-510XI Series Pure Performance Interior Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

- Alternate High Performance Option

Acrylic Epoxy Finish: Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Cubicles/Offices/Conference Room/Break Room - Gypsum Board Ceilings:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 6-4900XI Series Speedhide Interior Zero VOC Latex Sealer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

- Alternate Upgrade Option

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 9-900 Pure Performance Interior Latex Primer.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Flat Finish:  Pittsburgh Paints; 9-110XI Series Pure Performance Interior Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Cubicles/Offices/Conference Room/Break Room - Ferrous Metal Doors, Frames, and Miscellaneous Metals:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Cubicles/Offices/Conference Room/Break Room - Ferrous and Non-Ferrous Metal for High Performance Finish (Including Handrails):

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Cubicles/Offices/Conference Room/Break Room - Non-Ferrous Metal - Galvanized Surfaces and Aluminum:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Warehouse/Distribution - Concrete Masonry Units (CMU):

Acrylic Finish:  Two finish coats over a block filler.

Eggshell Finish:

Block Filler:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Eggshell Finish:  Pittsburgh Paints; 6-411ZV Series Speedhide Interior Zero VOC Latex Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate Upgrade Option

Acrylic Epoxy High Performance Finish:  Two finish coats over a block filler.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-100XI Perma-Crete Concrete Block & Masonry Surfacer/Filler.

Applied at a dry film thickness of not less than 8.0 mils (0.203 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Warehouse/Distribution - Masonry and Concrete Board Ceilings and Soffits:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Warehouse/Distribution - Non-Ferrous and Ferrous Decking Including Bar Joists:

Acrylic Dryfall Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 90-1912 Pitt-Tech Plus EP Interior/Exterior Primer Finish DTM Industrial Enamel.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Flat Finish:  Pittsburgh Paints; 6-725XI Series Speedhide Dry Fog Latex Flat.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Eggshell Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Eggshell Finish:  Pittsburgh Paints; 6-724XI Speedhide Super Tech WB Interior 100% Acrylic Dry-Fog Latex Eggshell.

Applied at a dry film thickness of not less than 1.8 mils (0.046 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Pitt-Tech Plus EP Interior/Exterior Primer Finish DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-727XI Series Speedhide Dry Fog Latex Semi-Gloss.

Applied at a dry film thickness of not less than 1.8 mils (0.046 mm).

Warehouse/Distribution - Ferrous Metal Doors, Frames, and Miscellaneous Metals:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Warehouse/Distribution - Ferrous Metals for High Performance Finish (Including Handrails):

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:  Pittsburgh Paints; 98E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Boiler Room/Physical Plant/Maintenance Room - Masonry and Concrete Walls:

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Eggshell Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Eggshell Finish:  Pittsburgh Paints; 16-1310 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Eggshell.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

- Alternate High Performance Option

Acrylic Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 98E-1/98E-100 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Gloss Finish:  Pittsburgh Paints; 198E-1/98E-98 Series Aquapon WB EP Two-Component Waterborne Epoxy Coating Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Boiler Room/Physical Plant/Maintenance Room - Masonry and Concrete Ceilings:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-70ZV Series Speedhide Interior Zero VOC Latex Flat.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Boiler Room/Physical Plant/Maintenance Room - Ferrous Metal Doors, Frames, and Miscellaneous Metals:

Acrylic Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate Upgrade Option

Acrylic Pre-Catalyzed Epoxy Finish:  Two finish coats over a primer.

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 90-1912 Series Pitt-Tech Plus EP Interior/Exterior DTM Industrial Primer.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 16-1510 Series Pitt-Glaze WB1 Interior Pre-Catalyzed Water-Borne Acrylic Epoxy Semi-Gloss.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Concrete Floors - Pigmented Coating System (Light Duty Foot Traffic to Medium/Heavy Duty Applications):

Gloss Epoxy Finish:  Two finish coats.

Gloss Finish:

Primer:  PPG Flooring; 912 LV Concrete Epoxy Primer/Sealer

Applied at a dry film thickness of not less than 6.0 mils (0.150 mm).

Gloss Finish:  PPG Flooring; 610 SL Self-Leveling Epoxy

Applied at a dry film thickness of not less than 5.0 mils (0.127 mm).

Concrete Floors - Decorative:

Pittsburgh Paints offers a full line of patching materials, basecoats, and high-performance floor finishes and has a coating solution for most surfaces. Contact your local Pittsburgh Paints Sales Representative to evaluate the project for a complete product recommendation.

Commercial Facility Exterior Paint Systems

Concrete and Masonry - Smooth Surfaces:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-610XI Series Speedhide Exterior House Paint Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Flat Finish- High Build Coating:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 4-22XI Series Perma-Crete High Build 100% Acrylic Topcoat Flat.

Applied at a dry film thickness of not less than 3.2 mils (0.081 mm).

Satin Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Satin Finish:  Pittsburgh Paints; 6-2045XI Series Speedhide Exterior House and Trim Acrylic Latex Satin.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Satin Finish - High Build Coating:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Satin Finish:  Pittsburgh Paints; 4-422 Series Perma-Crete High Build 100% Acrylic Topcoat Satin.

Applied at a dry film thickness of not less than 3.0 mils (0.076 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-900XI Series Speedhide Exterior House and Trim Acrylic Latex Paint Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Concrete and Masonry Vertical Surfaces - Textured:

Acrylic Texture Finish:  Two finish coats over a primer.

Fine Texture:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Fine Textured Flat Finish:  Pittsburgh Paints; 4-50 Series Perma-Crete 100% Acrylic Textured Coatings Flat.

Applied at a dry film thickness of not less than 6.8 mils (0.173 mm).

Medium Hard Texture:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Medium Hard Textured Flat Finish:  Pittsburgh Paints; 4-60 Series Perma-Crete 100% Acrylic Textured Coatings Flat.

Applied at a dry film thickness of not less than 6.8 mils (0.173 mm).

Medium Soft Texture:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Medium Soft Textured Flat Finish:  Pittsburgh Paints; 4-65 Series Perma-Crete 100% Acrylic Textured Coatings Flat.

Applied at a dry film thickness of not less than 6.8 mils (0.173 mm).

Coarse Texture:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Coarse Textured Flat Finish:  Pittsburgh Paints; 4-70 Series Perma-Crete 100% Acrylic Textured Coatings Flat.

Applied at a dry film thickness of not less than 6.8 mils (0.173 mm).

Concrete Unit Masonry (Porous Surfaces):

Acrylic Finish:  Two finish coats over a block filler.

Flat Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Flat Finish:  Pittsburgh Paints; 6-610XI Series Speedhide Exterior House Paint Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Flat Finish- High Build Coating:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Flat Finish:  Pittsburgh Paints; 4-22XI Series Perma-Crete High Build 100% Acrylic Topcoat Flat.

Applied at a dry film thickness of not less than 3.2 mils (0.081 mm).

Satin Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Satin Finish:  Pittsburgh Paints; 6-2045XI Series Speedhide Exterior House and Trim Acrylic Latex Satin.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Satin Finish - High Build Coating:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi Fill Acrylic Masonry Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Satin Finish:  Pittsburgh Paints; 4-422 Series Perma-Crete High Build 100% Acrylic Topcoat Satin.

Applied at a dry film thickness of not less than 3.0 mils (0.076 mm).

Semi-Gloss Finish:

Block Filler:  Pittsburgh Paints; 6-15XI Speedhide Interior/Exterior Hi-Fill Acrylic Masonry Block Filler.

Applied at a dry film thickness of not less than 7.0 mils (0.178 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-900XI Series Speedhide Exterior House and Trim Acrylic Latex Paint Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Plaster, Stucco, and EIFS Surfaces:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 6-610XI Series Speedhide Exterior House Paint Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Flat Finish - High Build Coating:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Flat Finish:  Pittsburgh Paints; 4-22XI Series Perma-Crete High Build 100% Acrylic Topcoat Flat.

Applied at a dry film thickness of not less than 3.2 mils (0.081 mm).

Satin Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Satin Finish:  Pittsburgh Paints; 6-2045XI Series Speedhide Exterior House and Trim Acrylic Latex Satin.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Satin Finish - High Build Coating:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Satin Finish:  Pittsburgh Paints; 4-422 Series Perma-Crete High Build 100% Acrylic Topcoat Satin.

Applied at a dry film thickness of not less than 3.0 mils (0.076 mm).

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 4-603XI Perma-Crete Interior/Exterior Alkali Resistant Primer.

Applied at a dry film thickness of not less than 1.2 mils (0.030 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-900XI Series Speedhide Exterior House and Trim Acrylic Latex Paint Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

Ferrous and Non-Ferrous Metals:

Primer is not required on shop-primed items.

Acrylic Finish:  Two finish coats over a rust-inhibitive primer.

Satin Finish:

Primer (if required):  Pittsburgh Paints; 90-1912 Pitt-Tech Plus EP Interior/Exterior Primer Finish DTM Industrial Enamel.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Satin Finish:  Pittsburgh Paints; 90-1710 Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Satin.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:

Primer (if required):  Pittsburgh Paints; 90-1912 Pitt-Tech Plus EP Interior/Exterior Primer Finish DTM Industrial Enamel.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 90-1610 Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Semi-Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:

Primer (if required):  Pittsburgh Paints; 90-1912 Pitt-Tech Plus EP Interior/Exterior Primer Finish DTM Industrial Enamel.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

Gloss Finish:  Pittsburgh Paints; 90-1510 Pitt-Tech Plus EP Interior/Exterior DTM Industrial Enamel Gloss.

Applied at a dry film thickness of not less than 2.0 mils (0.050 mm).

- Alternate High Performance Gloss Finish

Epoxy/Urethane Finish:  Two finish coats over epoxy primer.

Gloss Finish (High Performance):

Primer (if required):  PPG Protective and Marine Coatings; Amerlock 2 VOC High Solids Epoxy.

Applied at a dry film thickness of not less than 4.0 mils (0.10 mm).

Finish:  PPG Protective and Marine Coatings; Amershield VOC Polyester Acrylic Polyurethane Gloss.

Applied at a dry film thickness of not less than 3.0 mils (0.076 mm).

Wood - Siding, Trim, and Other Smooth Exterior Wood Surfaces:

Acrylic Finish:  Two finish coats over a primer.

Flat Finish:

Primer:  Pittsburgh Paints; 17-921XI Series Seal Grip 100 Percent Acrylic Universal Primer.

Applied at a dry film thickness of not less than 1.6 mils (0.041 mm).

Flat Finish:  Pittsburgh Paints; 6-610XI Series Speedhide Exterior House Paint Latex Flat.

Applied at a dry film thickness of not less than 1.5 mils (0.038 mm).

Satin Finish:

Primer:  Pittsburgh Paints;17-921XI Series Seal Grip 100 Percent Acrylic Universal Primer.

Applied at a dry film thickness of not less than 1.6 mils (0.041 mm).

Satin Finish:  Pittsburgh Paints; 6-2045XI Series Speedhide Exterior House and Trim Acrylic Latex Satin.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm)

Semi-Gloss Finish:

Primer:  Pittsburgh Paints; 17-921XI Series Seal Grip 100 Percent Acrylic Universal Primer:

Applied at a dry film thickness of not less than 1.6 mils (0.041 mm).

Semi-Gloss Finish:  Pittsburgh Paints; 6-900XI Series Speedhide Exterior House and Trim Acrylic Latex Paint Semi-Gloss.

Applied at a dry film thickness of not less than 1.4 mils (0.036 mm).

PART 3  EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

Coordination of Work:  Review other sections in which primers are provided to ensure compatibility of the total system for various substrates. On request, furnish information on characteristics of finish materials to ensure use of compatible primers.

Notify Architect about anticipated problems when using the materials specified over substrates primed by others.

If a potential incompatibility of primers applied by others exists, obtain the following from the primer applicator before proceeding:

Confirmation of primer's suitability for expected service conditions.

Confirmation of primer's ability to be top coated with materials specified.

PREPARATION

General:  Remove hardware and hardware accessories, plates, machined surfaces, lighting fixtures, and similar items already installed that are not to be painted. If removal is impractical or impossible because of size or weight of the item, provide surface-applied protection before surface preparation and painting.

After completing painting operations in each space or area, reinstall items removed using workers skilled in the trades involved.

Cleaning:  Before applying paint or other surface treatments, clean substrates of substances that could impair bond of the various coatings. Remove oil and grease before cleaning.

Schedule cleaning and painting so dust and other contaminants from the cleaning process will not fall on wet, newly painted surfaces.

Surface Preparation:  Clean and prepare surfaces to be painted according to manufacturer's written instructions for each substrate condition and as specified.

Provide barrier coats over incompatible primers or remove and reprime.

Cementitious Substrates:  Prepare concrete, brick, concrete masonry block, and cement plaster surfaces to be coated. Remove efflorescence, chalk, dust, dirt, grease, oils, and release agents. Roughen as required to remove glaze. If hardeners or sealers have been used to improve curing, use mechanical methods to prepare surfaces.

Use abrasive blast-cleaning methods if recommended by coating manufacturer.

Determine alkalinity and moisture content of surfaces by performing appropriate tests. If surfaces are sufficiently alkaline to cause the finish paint to blister and burn, correct this condition before application. Do not coat surfaces if moisture content exceeds that permitted in manufacturer's written instructions.

Wood Substrates:  Clean surfaces of dirt, oil, and other foreign substances with scrapers, mineral spirits, and sandpaper, as required. Smoothly sand surfaces exposed to view and dust off.

Scrape and clean small, dry, seasoned knots, and apply a thin coat of white shellac or other recommended knot sealer, before applying primer.

Immediately on delivery, prime edges, ends, faces, undersides, and backsides of wood to be coated.

After priming, fill holes and imperfections in the finish surfaces with putty or plastic wood filler. Sand smooth when dried.

Ferrous Metal Substrates:  Clean ungalvanized ferrous metal surfaces that have not been shop coated; remove oil, grease, dirt, loose mill scale, and other foreign substances. Use solvent or mechanical cleaning methods that comply with SSPC recommendations.

Blast-clean steel surfaces as recommended by coating manufacturer and according to SSPC-SP 6.

Treat bare and sandblasted or pickled clean metal with a metal treatment wash coat before priming.

Touch up bare areas and shop-applied prime coats that have been damaged. Wire brush, solvent clean, and touch up with same primer as the shop coat.

Non-Ferrous Metal Substrates:  Clean non-ferrous and galvanized surfaces according to manufacturer's written instructions for the type of service, metal substrate, and application required.

Remove pretreatment from galvanized sheet metal fabricated from coil stock by mechanical methods.

Material Preparation:  Carefully mix and prepare coating materials according to manufacturer's written instructions.

Maintain containers used in mixing and applying coatings in a clean condition, free of foreign materials and residue.

Stir materials before applying to produce a mixture of uniform density. Stir as required during application. Do not stir surface film into the material. Remove film and, if necessary, strain coating material before using.

Use only the type of thinners approved by manufacturer and only within recommended limits.

Tinting:  Tint each undercoat a lighter shade to simplify identification of each coat when multiple coats of same material are applied. Tint undercoats to match the color of the finish coat but provide sufficient differences in shade of undercoats to distinguish each separate coat.

APPLICATION

General:  Apply paint according to manufacturer's written instructions. Use applicators and techniques best suited for substrate and type of material being applied.

General:  Apply high-performance coatings according to manufacturer's written instructions.

Use applicators and techniques best suited for the material being applied.

Do not apply high-performance coatings over dirt, rust, scale, grease, moisture, scuffed surfaces, or conditions detrimental to forming a durable coating film.

Coating surface treatments and finishes are indicated in the coating system descriptions.

Provide finish coats compatible with primers used.

The term "exposed surfaces" includes areas visible when permanent or built-in fixtures, convector covers, grilles, covers for finned-tube radiation, and similar components are in place. Extend coatings in these areas, as required, to maintain system integrity and provide desired protection.

Application Procedures:  Apply coatings by brush, roller, spray, or other applicators according to manufacturer's written instructions.

The number of coats and film thickness required is the same regardless of application method.

Completed Work:  Match approved samples for color, texture, and coverage. Remove, refinish, or recoat work that does not comply with specified requirements.

FIELD QUALITY CONTROL

See Section 014000 - Quality Requirements, for general requirements for field inspection and testing.

Owner reserves the right to invoke the following test procedure at any time and as often as Owner deems necessary during the period when paint is being applied:

Owner will engage a qualified independent testing agency to sample paint material being used. Samples of material delivered to project will be taken, identified, sealed, and certified in the presence of Contractor.

Owner may direct Contractor to stop painting if test results show material being used does not comply with specified requirements. Contractor shall remove noncomplying paint from project site, pay for testing, and repaint surfaces previously coated with the noncomplying paint. If necessary, Contractor may be required to remove noncomplying paint from previously painted surfaces if, on repainting with specified paint, the two coatings are incompatible.

CLEANING

After completing painting, clean glass and paint spattered surfaces. Remove spattered paint by washing and scraping without scratching or damaging adjacent finished surfaces.

PROTECTION

Protect work of other trades, whether being painted or not, against damage from painting. Correct damage by cleaning, repairing, or replacing, and repainting, as approved by Architect.

Provide "Wet Paint" signs to protect newly painted finishes. After completing painting operations, remove temporary protective wrappings provided by others to protect their work.

After work of other trades is complete, touch up and restore damaged or defaced painted surfaces.

- END OF SECTION